# ABSTRACT

Modern network security faces an uneven challenge: once an enemy has a good map of target hosts and services through reconnaissance, it becomes much easier to use those resources. Traditional static IP addressing permits reconnaissance data to remain valid indefinitely, offering attackers a continuous and dependable intelligence foundation. This study introduces VANTA — Variable Network Topology Architecture — a prototype Moving Target Defence (MTD) system constructed on Software Defined Networking (SDN) that consistently undermines attacker reconnaissance by perpetually altering the virtual IP (VIP) identities presented to the network.

VANTA uses the Ryu SDN controller framework and the OpenFlow 1.3 protocol to catch all packet-in events at the control plane. When the system sees that communication has finished, such as an ICMP echo reply cycle, a TCP three-way handshake, or a UDP request-response exchange, it gives new virtual IP addresses to the host pairs that are taking part. This reply-triggered morphing is helped by three other methods: time-based periodic remorphing, packet-count-threshold morphing, and threat-triggered immediate morphing that starts when a port scan is detected.

The threat detection subsystem keeps an eye on TCP SYN packets and keeps track of unique attempts to access destination ports within configurable sliding windows. The system sees the activity as a port scan when the number of different ports contacted by a single source goes over a certain level in the observation window. It then sends out a threat event and, if the threat-triggered strategy is active, it immediately changes the virtual IP address, making all previously collected reconnaissance data useless within about one hundred milliseconds.

A Flask and a Socket.The IO web dashboard gives you real-time telemetry, such as topology visualisation, VIP mapping tables, morph event timelines, threat logs, and reports that you can export in CSV, JSON, and PDF formats. A login flow handles authentication and supports multi-factor authentication codes and access decisions that are aware of the device's trust level, in line with zero-trust principles.

Performance evaluation looks at the latency, throughput, CPU, and memory overhead that VIP morphing adds. Security evaluation evaluates the detection-to-morph time and the percentage of scan findings that become invalid following a morph event. This is called reconnaissance degradation. When compared to modern architectures like Zero Trust, microsegmentation, and BeyondCorp-style access models, VANTA's unique role as a dynamic network obfuscation layer that works with identity-centric security approaches becomes clearer.

Experimental results show that VANTA can successfully stop port scan reconnaissance within seconds of detection, with only a small amount of latency added in laboratory Mininet environments. The system's modular design, wide range of APIs, and tools for benchmarking that can be used again and again make it a good research platform for further MTD study.

Keywords: Moving Target Defense, Software Defined Networking, OpenFlow, Virtual IP Morphing, Port Scan Detection, Network Obfuscation, Cyber Deception, Zero Trust, Ryu Controller, Reconnaissance Invalidation

TABLE OF CONTENTS

| CHAPTER / SECTION | TITLE | PAGE |
| --- | --- | --- |
|  | ABSTRACT | v |
|  | TABLE OF CONTENTS | vi |
|  | LIST OF FIGURES | vii |
|  | LIST OF TABLES | viii |
|  | ABBREVIATIONS | ix |
| 1 | INTRODUCTION | 1 |
| 1.1 | Introduction to the Project | 2 |
| 1.2 | Problem Statement and Description | 3 |
| 1.3 | Motivation | 4 |
| 1.4 | Sustainable Development Goal | 5 |
| 2 | LITERATURE SURVEY | 6 |
| 2.1 | Overview of the Research Area | 7 |
| 2.2 | Existing Models and Frameworks | 8 |
| 2.3 | Research Gaps and Limitations | 11 |
| 2.4 | Research Objectives | 12 |
| 2.5 | Product Backlog and User Stories | 13 |
| 2.6 | Plan of Action — Project Roadmap | 15 |
| 3 | SPRINT PLANNING AND EXECUTION METHODOLOGY | 21 |
| 3.1 | Sprint I: SDN Foundation and VIP Morphing Engine | 21 |
| 3.2 | Sprint II: Threat Detection and Dashboard | 31 |
| 3.3 | Sprint III: Access Control and Deployment Profiles | 39 |
| 4 | SYSTEM DESIGN AND ARCHITECTURE | 45 |
| 4.1 | System Overview | 45 |
| 4.2 | Component Architecture | 47 |
| 4.3 | Data Flow and Packet Processing | 50 |
| 4.4 | Database and State Management | 52 |
| 5 | IMPLEMENTATION | 54 |
| 5.1 | Development Environment | 54 |
| 5.2 | Core Module Implementation | 55 |
| 5.3 | Attack Simulation Suite | 60 |
| 5.4 | Benchmarking Suite | 62 |
| 6 | RESULTS AND DISCUSSIONS | 64 |

| CHAPTER / SECTION | TITLE | PAGE |
| --- | --- | --- |
| 6.1 | Security Evaluation | 64 |
| 6.2 | Performance Evaluation | 66 |
| 6.3 | Comparative Analysis | 69 |
| 7 | CONCLUSION AND FUTURE ENHANCEMENT | 72 |
|  | REFERENCES | 75 |
| A | APPENDIX A — Core Code Listings | 78 |
| B | APPENDIX B — Plagiarism Report | 80 |

LIST OF FIGURES

| Figure No. | Title | Page |
| --- | --- | --- |
| 1.1 | Network reconnaissance attack chain in static addressing environments | 3 |
| 2.1 | Taxonomy of Moving Target Defense techniques | 8 |
| 2.2 | SDN architecture showing control plane separation | 9 |
| 2.3 | Timeline of key MTD research milestones | 10 |
| 3.1 | VANTA system architecture overview | 22 |
| 3.2 | VIP morphing workflow — reply-triggered scenario | 25 |
| 3.3 | Port scan detection state machine | 28 |
| 3.4 | MTD morphing workflow — five-phase model | 34 |
| 3.5 | Threat detection and response timeline | 36 |
| 4.1 | Complete component architecture of VANTA | 47 |
| 4.2 | Packet processing pipeline in the controller | 50 |
| 4.3 | VIP pool management lifecycle | 53 |
| 5.1 | Morphing strategy comparison summary | 58 |
| 5.2 | Attack simulator workflow | 61 |
| 6.1 | Reconnaissance degradation percentage across strategies | 65 |
| 6.2 | Latency overhead comparison with and without MTD | 67 |
| 6.3 | CPU and memory utilization over a 60-second monitoring window | 68 |
| 6.4 | VANTA vs modern security architectures — radar chart | 70 |

LIST OF TABLES

| Table No. | Title | Page |
| --- | --- | --- |
| 2.1 | Summary of related MTD literature — key findings and limitations | 9 |
| 2.2 | Comparison of SDN controller frameworks | 10 |
| 2.3 | Research gap analysis matrix | 11 |
| 2.4 | Product backlog with user stories and acceptance criteria | 13 |
| 3.1 | Sprint I deliverables and completion status | 23 |
| 3.2 | Morphing strategy configuration parameters | 26 |
| 3.3 | Sprint II deliverables and completion status | 33 |
| 3.4 | Threat detection threshold configuration | 36 |
| 3.5 | Sprint III deliverables and completion status | 40 |
| 4.1 | API endpoint specification | 49 |
| 4.2 | VIP pool addressing scheme | 52 |
| 5.1 | Development environment and tool versions | 54 |
| 5.2 | Attack simulation parameters | 60 |
| 5.3 | Benchmark test matrix | 62 |
| 6.1 | Reconnaissance degradation results per morphing strategy | 65 |
| 6.2 | Latency statistics with and without MTD active | 67 |
| 6.3 | VANTA vs modern architecture scoring matrix | 70 |
| 6.4 | Security and performance trade-off summary | 71 |

ABBREVIATIONS

| Abbreviation | Full Form |
| --- | --- |
| API | Application Programming Interface |
| ARP | Address Resolution Protocol |
| CSV | Comma Separated Values |
| DDoS | Distributed Denial of Service |
| DPID | Datapath Identifier |
| EMU | English Metric Unit |
| FIN | TCP Finish Flag |
| HTTP | Hypertext Transfer Protocol |
| ICMP | Internet Control Message Protocol |
| IDS | Intrusion Detection System |
| IP | Internet Protocol |
| JSON | JavaScript Object Notation |
| MAC | Media Access Control |
| MFA | Multi-Factor Authentication |
| MTD | Moving Target Defense |
| NIST | National Institute of Standards and Technology |
| OF | OpenFlow |
| OFPP | OpenFlow Physical Port |
| OVS | Open vSwitch |
| PDF | Portable Document Format |
| REST | Representational State Transfer |
| RTT | Round Trip Time |
| SDN | Software Defined Networking |
| SYN | TCP Synchronize Flag |
| TCP | Transmission Control Protocol |
| TTL | Time To Live |
| UDP | User Datagram Protocol |
| VIP | Virtual Internet Protocol (address) |
| VM | Virtual Machine |
| ZTA | Zero Trust Architecture |

CHAPTER 1 INTRODUCTION

## Introduction to the Project

The rise in the number of devices that can connect to the Internet and the growing intelligence of cybercriminals have put a lot of stress on traditional network security architectures. Firewalls, intrusion detection systems, and access control lists are examples of traditional perimeter-based defences. They all work on the idea that the network boundary is stable and well-defined. This assumption is quickly becoming less true in the age of cloud computing, remote workforces, and distributed microservices, where the idea of a perimeter no longer makes sense.

One of the main weaknesses of static network topologies is that they are easy to guess. When IP addresses, service ports, and protocol stacks stay the same over time, an enemy who successfully completes a reconnaissance pass gets a network map that stays useful forever. This intelligence advantage lets people run planned, low-noise campaigns that target specific people for long periods of time, well below the detection levels set for high-rate attacks.

Moving Target Defence (MTD) is a new way of doing things that directly solves this problem of predictability. Instead of trying to build static defences that can't be broken, MTD changes the properties of the system that face attacks all the time. This makes it so that reconnaissance data expires faster than it can be used. MTD techniques used at different network layers include changing virtual IP addresses, randomising port assignments, modifying protocol stacks, and rotating code variations.

VANTA, or Variable Network Topology Architecture, is a research prototype that uses IP-layer MTD over a Software Defined Network. VANTA uses the programmatic control of OpenFlow 1.3 and the Ryu SDN controller to create a VIP morphing engine that constantly moves virtual IP addresses around based on communication events, time intervals, packet counts, or threats that are found. The project adds a working controller implementation, a threat detection subsystem, a web dashboard that works in real time, an attack simulation suite, and a performance benchmarking suite.

## Problem Statement and Description

Static network addressing gives enemies a solid base of information. A port scanner can find open ports on a target and keep that information useful until the target's configuration changes. In production environments, this could take days, weeks, or never. Advanced persistent threat (APT) actors take advantage of this long validity window by doing slow, careful reconnaissance before launching attacks that are very specific.

The specific issue tackled in this research is articulated as follows: in the context of a network of hosts with IP addresses accessible to external or internal adversaries, devise a system that perpetually alters the IP identities exposed to the network, ensuring that reconnaissance data collected at any moment becomes obsolete within a manageable timeframe, while maintaining standard communication semantics for authorised users.

To find a solution, a number of smaller problems need to be solved. The VIP mapping layer needs to make sure that packets sent to virtual IPs are always changed to real IPs without any packet loss at the data plane. The morphing trigger logic needs to find a balance between security (frequent morphing shortens validity windows) and performance (each morph event makes existing flow table entries invalid, which increases the load on the controller). The threat detection subsystem needs to be able to tell the difference between real scanning behaviour and normal traffic that goes through many ports. Last but not least, the operator interface needs to give enough telemetry for people to keep an eye on things without giving away private real-IP topology information.

## Motivation

This research is motivated by three converging trends. First, more and more businesses and data centers are using SDN. This makes a programmable substrate that can be used to construct adaptive network behaviours without the need for special hardware. Second, the increasing amount of MTD research shows that IP randomisation clearly makes reconnaissance less useful. However, most current proposals either work at the OS/hypervisor level (which means changing the host) or are only tested in simulation (which doesn't show how networks really work). Third, the fact that there are open-source tools like Ryu, Mininet, and Open vSwitch makes it possible to build fully functional SDN-based MTD systems at a low cost.

In terms of context, VANTA is aimed at the reconnaissance phase of the cyber kill chain. The system makes reconnaissance outputs go bad quickly, which means that enemies have to either accept bad intelligence (which makes attacks noisier and more likely to be caught) or keep doing reconnaissance (which costs more resources for the attack). Defenders profit from both outcomes. The system doesn't try to stop reconnaissance, because that would be impossible to do consistently. Instead, it tries to make the results go away quickly.

## Sustainable Development Goal

This project fits with the UN's Sustainable Development Goal 9: Infrastructure, Industry, and Innovation. The research specifically helps make infrastructure more resilient and encourages new ideas in network security. This work supports the democratisation of advanced security capabilities by creating open-source, reproducible research tools for adaptive cyber defence. This allows schools, small businesses, and organisations in developing countries to try out and use MTD techniques without having to pay for proprietary licenses.

The project also helps SDG 16: Peace, Justice, and Strong Institutions by improving our scientific understanding of cyber resilience, which is necessary for public institutions to have safe digital governance frameworks.

# CHAPTER 2 LITERATURE SURVEY

## Overview of the Research Area

In 2009, a DARPA-funded project formally introduced Moving Target Defence as a research area. The goal was to change the way we think about cybersecurity from static, predictable systems to dynamic, unpredictable ones. Jajodia et al. (2011) say that defenders have a systematic asymmetry: they have to protect all of their assets all the time, but attackers only have to find and use one weakness at the right time. MTD tries to bring things back into balance by making the time-value of attacker intelligence less important.

The research landscape covers many levels of the network stack. At the application layer, different kinds of software and randomising the instruction set keep memory exploitation techniques from working on all copies of the same application. IP address randomisation, port hopping, and topology obfuscation at the network layer, which is the main focus of this work, stop enemies from making stable network maps. Protocol obfuscation and traffic morphing hide communication patterns from deep packet inspection systems at the data layer.

Software Defined Networking makes it possible for network-layer MTD to work because its control plane separation lets a central controller change data-plane forwarding rules programmatically, without having to physically change anything. OpenFlow, the most popular SDN southbound protocol, has flow table manipulation primitives that can rewrite IP addresses at line rate on cheap switches.

## Existing Models and Frameworks

A significant corpus of literature investigates IP-layer MTD within the context of SDN. We look at the most important prior art, which is grouped by method and place of publication.

One of the first SDN-based IP randomisation systems was OpenFlow Random Host Mutation (OF-RHM), which Jafarian et al. (2012) came up with. OF-RHM gives out random IP addresses from a certain virtual address space and changes them every so often. The system shows that scan completion rates go down when mutation intervals are shorter than the normal scan time. Its tests are only done with synthetic traffic generators.

Sharma et al. (2015) expanded the IP randomisation notion by introducing a proactive network topology shuffling methodology. Their framework not only changes IP addresses, but also logical network paths, which makes path-dependent attacks less reliable. The overhead analysis in their evaluation shows that topology reshuffling costs more CPU time for the controller than pure IP mutation. This is why IP morphing and topology mutation need to be kept separate in future systems.

Luo et al. (2015) proposed MT6D, an MTD system operating over IPv6 that utilizes the extensive IPv6 address space to assign hosts random interface identifiers with a high degree of temporal precision. MT6D does a great job of making it hard to guess addresses, but it can’t be used in enterprise networks that mostly use IPv4 because it needs IPv6. VANTA focuses on IPv4 environments for this reason.

Debroy et al. (2016) assessed the  of effectiveness of MTD against advanced threats and concluded that threat-triggered morphing—initiating address mutation in direct response to identified scanning events rather than following a predetermined schedule—exceeds purely proactive strategies in scenarios marked by infrequent yet sophisticated attackers. This discovery directly led to the threat-triggered technique method used in VANTA.

| Reference | Approach | Trigger | Evaluation | Limitations |
| --- | --- | --- | --- | --- |
| Jafarian et al. (2012) | OF-RHM IP randomization | Time-based | Simulation | No threat-adaptive response |
| Sharma et al. (2015) | Topology shuffling | Time-based | Mininet testbed | High controller overhead |
| Luo et al. (2015) | MT6D IPv6 randomization | Time-based | Physical testbed | IPv6 only |
| Debroy et al. (2016) | Threat-adaptive MTD | Scan detection | Simulation | No SDN integration |
| Meier et al. (2018) | SDN-based port hopping | Time + threat | Mininet | No dashboard or API |
| Zhang et al. (2019) | ML-guided MTD timing | Predictive | Simulation | Training data dependency |
| Cho et al. (2020) | MTD + deception | Event-based | Emulation | High complexity |
| VANTA (this work) | VIP morphing + detection | Multi-strategy | Mininet + benchmarks | Lab-scale evaluation |

Table 2.1: Summary of related MTD literature — key findings and limitations

Meier et al. (2018) used port-number hopping over an SDN substrate to show that service-port randomisation works well with IP address randomisation to stop service-specific scanning tools. Their system uses both time-based and threat-triggered port hopping, but it doesn't have a management API or a real-time monitoring interface that users can use. With its Socket, VANTA fills this need.IO telemetry and a dashboard based on Flask.

Zhang et al. (2019) released an ML-guided adaptive MTD that uses past information about past threats to find morphing intervals. This method is better than fixed interval morphing at finding a balance between security and performance, but it need a training dataset of attack patterns, which makes it hard to use in places where labelled threat data isn’t. VANTA intentionally avoids ML dependencies to make it easier to move and reproduce.

## Research Gaps and Limitations Identified from Literature

The analysis of the reviewed literature uncovers multiple persistent gaps that this study seeks to rectify.

- No integrated real-time dashboards: Earlier SDN-MTD prototypes only showed results through log files or simulation output. None of them have a real-time operator interface that can be accessed through a browser, with live topology visualisation, morph event streaming, and reports that can be exported.

- Single-strategy evaluation: Most previous studies only look at one morphing strategy, usually time-based. They don't compare reply-triggered, packet-count, and threat-triggered variants in the same experimental conditions.

- Limited threat detection integration: Debroy et al. (2016) theoretically endorsed threat-triggered MTD; however, only a handful of SDN controller implementations that directly incorporate a port-scan detector into the packet processing pipeline with an immediate morph response.

- No tools for reproducible benchmarking: Earlier papers show results for latency and throughput results but don't include the scripts used to get them, so they can’t be produced. VANTA sends benchmark_suite.py and attack_simulator.py as high-quality project files.

- Weak operational maturity: Previous SDN-MTD prototypes don't have usefull features for production, like authentication, multi-factor access control, deployment profile selection, or export features that are useful for production.

## Research Objectives

This research aims to address the highlighted shortcomings by pursuing the following key objectives:

- Build a working SDN-based MTD controller with OpenFlow 1.3 and Ryu that can allocate VIPs, process packet-ins, and change IP pairs without breaking up real communication flows.

- Design and add a port-scan detection subsystem that works in real time within the controller's packet processing pipeline and starts defensive morphing when a scan is confirmed.

- Try out and compare five morphing strategies under the same experimental conditions: reply-triggered, time-based, packet-count-based, randomized-interval, and threat-triggered.

- Make a web dashboard that displays telemetry data from Socket.IO, VIP mapping, morph event history, and the ability to export data in different formats.

- Create a set of attack simulations and benchmarks that can be used over and over again to measure security improvements and performance costs.

- Look at how VANTA fits in with modern security architectures like Zero Trust, microsegmentation, and BeyondCorp. Also, think about where it could work with other technologies.

## Product Backlog and User Stories

| Story ID | User Story | Acceptance Criteria | Priority | Sprint |
| --- | --- | --- | --- | --- |
| US-01 | As a network admin, I want IP addresses to morph after each communication so attacker reconnaissance data expires quickly. | Morph event logged within 200ms of reply detection. | High | 1 |

| Story ID | User Story | Acceptance Criteria | Priority | Sprint |
| --- | --- | --- | --- | --- |
| US-02 | As a security analyst, I want to see live VIP mappings on a dashboard so I understand current network state. | Dashboard updates within 2 seconds of any mapping change. | High | 2 |
| US-03 | As an operator, I want the system to detect port scans and react immediately. | Threat event emitted and morph triggered within 500ms. | High | 2 |
| US-04 | As a researcher, I want to configure different morphing strategies via an API. | POST /api/strategy accepts and applies strategy parameters. | Medium | 2 |
| US-05 | As an auditor, I want to export morph history as CSV/JSON/PDF. | All three export formats downloadable from the dashboard. | Medium | 3 |
| US-06 | As an admin, I want login and MFA to protect dashboard access. | Sessions require valid credentials; admin actions require MFA. | High | 3 |
| US-07 | As a researcher, I want benchmark scripts that measure RTT and throughput overhead. | benchmark_suite.py produces structured JSON results. | Medium | 3 |
| US-08 | As a developer, I want unit tests covering core morphing logic. | Test suite passes with zero failures on core modules. | Medium | 3 |

Table 2.4: Product backlog with user stories and acceptance criteria

## Plan of Action — Project Roadmap

The project is organized into three two-week sprints following an Agile Scrum methodology. Each sprint targets a coherent set of user stories and concludes with a working software increment that is demonstrable to stakeholders.

| Phase | Duration | Deliverables | Success Criteria |
| --- | --- | --- | --- |
| Sprint I | Weeks 1-2 | Ryu controller, VIP pool, morph logic, OpenFlow integration | h1 ping h2 triggers morph event in Mininet |
| Sprint II | Weeks 3-4 | Threat detector, Flask dashboard, Socket.IO telemetry, strategy API | Port scan triggers immediate morph; dashboard updates in real-time |
| Sprint III | Weeks 5-6 | Auth/MFA, export APIs, benchmarking suite, attack simulator, unit tests | Full test suite passes; exports download correctly |
| Evaluation | Weeks 7-8 | Security and performance experiments, comparative analysis, report | Documented results in structured JSON; report completed |

Table 2.5: Sprint roadmap and success criteria

# CHAPTER 3 SPRINT PLANNING AND EXECUTION METHODOLOGY

## Sprint I: SDN Foundation and VIP Morphing Engine

### Objectives with User Stories of Sprint I

Project Sprint I is concerned with laying the necessary groundwork for VIP morphing using SDN. The central aim of Sprint I is to design a functional Ryu-based controller for OpenFlow 1.3 that is capable of sniffing packets in order to assign virtual IP addresses to discovered hosts and modify IP pairs once communications cease.

The sprint's goals are User Stories US-01 (IP morphing after communication is finished) and the basic parts of US-02 (initial topology awareness). For Sprint I to be accepted, there must be a working Mininet scenario where a ping between two hosts causes a verifiable VIP remapping that is logged to the controller console.

| Task | Description | Estimated Hours | Status |
| --- | --- | --- | --- |
| T1-01 | Set up Ryu controller skeleton with OpenFlow 1.3 | 4 | Done |
| T1-02 | Implement L2 MAC learning and flood-safe forwarding | 6 | Done |
| T1-03 | Design VIP pool data structure and allocator | 4 | Done |
| T1-04 | Implement real_to_virtual and virtual_to_real mappings | 4 | Done |
| T1-05 | Implement morph_ip_pair with guard and pool management | 6 | Done |
| T1-06 | Implement ICMP packet handler with reply-triggered morph | 5 | Done |
| T1-07 | Implement TCP packet handler with 3-way handshake detection | 8 | Done |
| T1-08 | Implement UDP packet handler with request-reply detection | 5 | Done |
| T1-09 | Integration testing in Mininet single,3 topology | 4 | Done |

Table 3.1: Sprint I task breakdown and completion status

### Functional Document — Sprint I

The VIP morphing engine works on the idea of address virtualisation. Every host that talks to the VANTA-controlled switch gets a virtual IP address that is different from its real IP address. It is possible to send packets with virtual destination IPs to real IPs without anyone knowing. When packets leave real IPs, they are sent to virtual IPs. The API for modifying flow in Ryu helps in this conversion because it supports the addition of per-flow rewriting instructions in the OpenFlow switch.

There are a total of 254 addresses in the network of 192.168.100.0/24 that make up the VIP pool. This is contrary to the addresses of Mininet hosts, which are in the range of 10.0.0.0/24. The controller allocates a VIP to the host on detection of the latter through either an ARP packet or IP packet. When a morph event happens, the old VIP goes back to the pool and a new VIP is drawn. All relevant controller state is updated at the same time to avoid temporary forwarding inconsistencies.

Morph guard plays an important role in implementation. Before proceeding, morph_ip_pair method ensures that both IPs being considered have VIP assignments.. If one of them is missing, like when a morph is triggered before one endpoint sends its first packet, the function gives out the missing VIPs and returns without morphing, letting the next trigger work. This guard stops silent failures and makes sure that the controller's state stays the same.

### Architecture Document — Sprint I

Figure 3.1: VANTA system architecture overview — control plane components and data flows

The architecture divides concerns into four main parts. The app_manager is the parent of the Ryu app (UltimateMTDController).RyuApp sets up event handlers for switch connection events (CONFIG_DISPATCHER) and packet-in events (MAIN_DISPATCHER). The VIP allocator is in charge of the real_to_virtual and virtual_to_real dictionaries and the virtual address pool. Protocol handlers handle_icmp, handle_tcp, and handle_udp examine specific layers of network protocols and monitor the state of flows in order to detect triggers. Flask application is running inside a daemon thread and accesses the state dictionaries of the controller through controller_instance variable.

Communication between Ryu event loop (managed by eventlet greenlet scheduler) and Flask thread is achieved using shared data structures within Python codebase, which are protected from concurrent access by the GIL. In production environment, explicit locking would be required, but for our prototype application GIL is enough since the Flask-based monitoring dashboard has few concurrent users.

### Outcome of Objectives — Sprint I

Sprint I gave a working controller that met the acceptance criteria. With one switch and three hosts in Mininet, the following order was checked:

- When hosts h1 and h2 first talk to each other, they are given VIPs 192.168.100.47 and 192.168.100.183.

- h1 pings h2; the ICMP echo request is stopped, VIPs are confirmed, and the ICMP tracker is updated.

- The controller gets the echo reply from h2, which starts the reply-triggered strategy and calls morph_ip_pair.

- The VIP of h1 changes from 192.168.100.47 to 192.168.100.201; h2's VIP goes from 192.168.100.183 to 192.168.100.88.

- The morph event is recorded with a timestamp, protocol, old VIPs, new VIPs, and a trigger type of "reply."

The morph latency, which is the time between receiving a packet and sending out a morph event log, was always less than 50ms in the Mininet environment, which is well below the target of 200ms.

### Sprint I Retrospective

The expected duration of Sprint I was completed in two weeks as planned. The logic of morph guards posed the greatest challenge during implementation. Initially, morph events were abruptly terminated in case one end point had not responded, leading to unpredictable gaps in the logs of morph events in case of uneven network traffic. The use of the guard and allocate strategy solved this challenge in a systematic manner.

A major challenge was that of maintaining the flow tables consistency in relation to morph events. It can prove difficult to initiate new flows in the presence of existing ones with the same virtual IP address. The approach involved the use of low idle timeouts for flow entries. In such a case, it will be easier for stale flow entries to automatically expire rather than deleting them. This will work well in the prototype, but should be refined in case of actual installations handling heavy network traffic.

## Sprint II: Threat Detection and Dashboard

### Objectives with User Stories of Sprint II

The threat detection module and the live web dashboard are the desired output for Sprint II. The key outputs that have to be accomplished include the ThreatDetector class that has the port-scan detection algorithm, the Flask server with Socket.IO telemetry capabilities, and the strategy configuration API. The sprint backlog consists of User Stories US-02 (live dashboard), US-03 (scan detection and response), and US-04 (strategy API).

| Task | Description | Estimated Hours | Status |
| --- | --- | --- | --- |
| T2-01 | Implement ThreatDetector with sliding-window port-scan detection | 8 | Done |
| T2-02 | Integrate threat detector into TCP packet handler | 4 | Done |
| T2-03 | Implement Flask application skeleton with login | 6 | Done |
| T2-04 | Implement Socket.IO real-time event emission | 6 | Done |

| Task | Description | Estimated Hours | Status |
| --- | --- | --- | --- |
| T2-05 | Build VANTA dashboard HTML with charts and topology vis | 10 | Done |
| T2-06 | Implement /api/stats, /api/mappings, /api/strategy endpoints | 6 | Done |
| T2-07 | Implement force morph API endpoint | 2 | Done |
| T2-08 | Implement MTDStatistics class with morph history | 4 | Done |
| T2-09 | Integration testing of threat-triggered morph workflow | 5 | Done |

Table 3.2: Sprint II task breakdown and completion status

### Functional Document — Sprint II

The threat detection subsystem uses a sliding-window algorithm to figure out if TCP SYN packets are being used for reconnaissance. The detector keeps track of a different set of destination ports that were contacted inside the observation window (by default, 10 seconds) for each pair of source IP and destination IP. It also keeps track of the window start timestamp. The detector checks to see if the window has expired when a new SYN packet comes in. If it has, the port set and timestamp are reset. A new destination port is added to the set. If the size of the set is equal to or greater than the threshold (which is 5 distinct ports by default), a port scan is declared.

When the scan is declared, the ThreatEvent is added to the detector's threat log, the port set is reset (so the same attacker might trigger it again after the next morph), and the function returns True to the caller. If the threat-triggered strategy is active, the TCP handler checks this return value and then calls morph_ip_pair right away for the attacker and victim IP pair.

Figure 3.2: Threat detection and response timeline — from scan onset to VIP remorphing

The web dashboard is a single-page application that is built from an inline HTML template string in the controller. This makes the controller self-contained because it doesn't need any external template files. The dashboard uses Socket to connect to the server.IO gets three types of events: stats_update (a full statistics dictionary), mappings_update (a list of current VIP mappings), and threat_detected (a notification of a new threat event). Chart.js makes protocol distribution doughnuts and morph trigger bar charts, while vis-network makes the SDN topology graph.

### Architecture Document — Sprint II

Figure 3.3: MTD morphing workflow — five phases from reconnaissance onset to recon expiry

The threat detection integration uses a non-blocking pattern in the TCP packet handler. The scan detector is called for every SYN packet, no matter how the strategy is set up. If a scan is found and the threat-triggered strategy is on, morphing happens and the packet handler returns early. This means that the SYN tracking entry is not made because morphing makes the session context invalid. The handler logs the threat event if the scan is found but threat-triggered is not active. It then continues with normal SYN tracking. This design makes sure that threat events are always logged, no matter what strategy is in use.

### Outcome of Objectives — Sprint II

The port-scan detector was tested against attack_simulator.py running in Mininet. The detector reliably fired after the fifth unique port was probed, sending a threat event to the dashboard within 200ms and starting morphing within 300ms of the fifth SYN packet's arrival. The scan threshold was set to 5 ports and the window to 10 seconds.

The dashboard looks good in both Chromium and Firefox. It shows live topology graphs, protocol count rings, morph trigger bar charts, and the VIP mapping table. Socket.IO latency (the time it takes for a controller event to be sent and the dashboard to be drawn) was measured to be between 15 and 50 milliseconds over localhost, which is within the 2-second update goal.

### Sprint II Retrospective

The concurrent threading model was the biggest problem in Sprint II. Eventlet greenlets are used in the Ryu event loop, whereas Flask utilises a regular threading model with eventlet monkeypatching applied via socketio.run. To make sure that shared state was accessed safely, all shared dictionary accesses had to be carefully checked. We fixed a small bug where the threat_detector.threats list was being added to by both the Ryu greenlet and a Flask endpoint at the same time. To do this, we made the list append atomic (a single append operation in CPython is protected by the GIL) and only allowed the Ryu greenlet to change it.

## Sprint III: Access Control, Export, and Benchmarking

### Objectives with User Stories of Sprint III

Sprint III adds the following operational maturity features: authentication with support for MFA, evaluation of a zero-trust access policy, multi-format export APIs, the attack simulation suite, the performance benchmarking suite, and the core logic unit test suite. The sprint is based on User Stories US-05 (exports), US-06 (auth/MFA), US-07 (benchmarks), and US-08 (unit tests).

| Task | Description | Estimated Hours | Status |
| --- | --- | --- | --- |
| T3-01 | Implement Flask-Login with session management | 5 | Done |
| T3-02 | Implement MFA code verification and device trust context | 4 | Done |
| T3-03 | Implement ZeroTrustAccessPolicy evaluation | 6 | Done |
| T3-04 | Implement CSV export endpoint | 2 | Done |
| T3-05 | Implement JSON export endpoint | 1 | Done |
| T3-06 | Implement PDF export with ReportLab | 5 | Done |
| T3-07 | Build attack_simulator.py with 4 attack modes | 8 | Done |
| T3-08 | Build benchmark_suite.py with latency, throughput, resource tests | 8 | Done |
| T3-09 | Extract core logic to vanta_core package | 6 | Done |
| T3-10 | Write unit tests for MorphingStrategy and ThreatDetector | 6 | Done |

Table 3.3: Sprint III task breakdown and completion status

### Functional Document — Sprint III

Flask-Login is used by the authentication subsystem to manage sessions, and passwords are stored as Werkzeug-generated bcrypt hashes. The login form asks for your username, password, MFA code, device ID, and device trust level. When the credentials are verified, the MFA code is checked against a reference code that can be changed (by default, it is set to a demo value that can be changed through an environment variable). The session keeps the device ID and trust level so that the access policy may be checked later.

Before the request is processed, all APIs invoke the _enforce_access() method. This method uses the current session and request headers to construct an AccessContext object, which it passes to ZeroTrustAccessPolicy.evaluate(), storing the resulting policy evaluation in the session. In determining the risk and trust scores, the access policy considers the role of the user, the trustworthiness of the device, whether MFA is enabled, the operating mode, and the remote access markers.

The export subsystems show how useful the morph_history list in the MTDStatistics class is. The CSV exporter publishes all morph events as a CSV stream that per RFC 4180. The JSON exporter turns the whole output of get_stats_dict() into a string. The PDF exporter uses ReportLab's Platypus framework to put together a formatted document that includes a table of summary statistics and a table of recent morph events.

The attack simulation suite (attack_simulator.py) uses the Scapy packet building module to set up four different attack scenarios. The port_scan module can do three types of scans: SYN scan (half-open), TCP connect scan (full connect), and XMAS scan (FIN-PSH-URG). The syn_flood module makes a lot of TCP SYN packets with various source IPs and ports to mimic a volumetric denial-of-service attack. The ping_flood module sends a lot of ICMP echo requests very quickly.

The reconnaissance module includes ping sweep, service enumeration, and TTL-based OS fingerprinting.

### Architecture Document — Sprint III

Figure 3.4: Morphing strategy comparison — trigger conditions, overhead, and security benefit

The vanta_core package takes logic from the monolithic controller and puts it into modules that can be imported. The config module defines RuntimeConfig and load_runtime_config(), which reads configuration from environment variables and sets safe defaults. The defence module consists of two classes, MorphingStrategy and ThreatDetector. These are the same as in the controller module so that testing can be done without the need for Ryu. Classes like AccessContext, AccessDecision, and ZeroTrustAccessPolicy belong to the access_control module. The deployment module includes the definitions for DeploymentProfile and load_deployment_profile().

### Outcome of Objectives — Sprint III

We sent out all of the Sprint III user stories. The credential verification process correctly authenticates users, ensures multi-factor and device authentication, and restricts access to API endpoints only for administrators, according to policy decisions. It is possible to download all three export formats without any errors through the dashboard interface. The attack simulation tool generates structured JSON files that are compatible with the benchmarking tool. The unit test suite tests the MorphingStrategy trigger logic, the ThreatDetector scan detection, the access policy evaluation, and the loading of runtime configuration. It covers 87% of the lines in the vanta_core modules.

### Sprint III Retrospective

Sprint III confirmed that the vanta_core extraction was useful: it was easy to write unit tests for the extracted classes, but testing the monolithic controller required mocking Ryu's event infrastructure. The biggest technical debt found is that flow entries are not cleaned up when morph events happen. The current implementation depends on the expiration of idle timeouts. This means that there is a window of up to 30 seconds during which old flow entries could cause packets to go to the wrong place for connections that are still active after a morph event. To fix this in the future, there would need to be explicit flow deletion API calls that work with morph events.

# CHAPTER 4 SYSTEM DESIGN AND ARCHITECTURE

## System Overview

VANTA's deployment architecture is designed for a Linux environment that uses Mininet network emulation, Open vSwitch (OVS) as the data-plane switching substrate, and Ryu as the SDN controller. There is only one OpenFlow switch that connects all the emulated hosts in the logical topology. The Ryu controller listens for OpenFlow connections on TCP port 6653, and the Flask dashboard serves on TCP port 5000.

The system sees any host that is not in the protected host set as a possible enemy from a threat model point of view. The VIP morphing mechanism works for all pairs of hosts that are communicating, even those that are on the same network. This is because reconnaissance is a threat no matter where it comes from, whether it's from outside the network or from a compromised internal host.

The system works on three different levels: the data plane (OVS forwarding hardware/software), the control plane (Ryu controller application logic), and the administration plane (Flask dashboard and REST API). The SDN paradigm gave rise to the idea of separating these planes, which is a basic architectural principle. This lets the management plane change the behaviour of the data plane without having to directly access the hardware.

## Component Architecture

The controller exposes the following API surface to the management plane:

| Endpoint | Method | Auth | Purpose |
| --- | --- | --- | --- |
| / | GET | Required | Render live dashboard HTML |
| /login | GET/POST | None | Credential and MFA verification |
| /logout | GET | Required | Clear session and redirect |
| /api/health | GET | None | Service health and deployment mode |
| /api/stats | GET | Required | Full controller statistics dict |
| /api/mappings | GET | Required | Current VIP mapping table |
| /api/strategy | GET/POST | Required | Read or configure morphing strategy |
| /api/morph/force | POST | Required | Force immediate morphing of all pairs |
| /api/access/context | GET | Required | Current access context and policy decision |
| /api/deployment/profile | GET | Required | Active deployment profile metadata |
| /api/export/csv | GET | Required | Download morph history as CSV |
| /api/export/json | GET | Required | Download statistics as JSON |
| /api/export/pdf | GET | Required | Download formatted PDF report |

Table 4.1: Complete API endpoint specification with authentication requirements

## Data Flow and Packet Processing

The packet processing pipeline starts when an OVS switch that is linked to the Ryu controller through an OpenFlow 1.3 channel gets a packet that doesn't match any of the flow table entries that are already there. The switch sends a packet-in message to the controller. This message has the packet payload and information about the ingress port.

The process is done using the packet_in_handler function of UltimateMTDController. Initially, the L2 MAC learning table is updated where the source MAC address is mapped with the ingress port. After that, the output port is identified. If the destination MAC address is known, the appropriate port will be used; otherwise, the packet will be flooded across all available ports.

Subsequently, the packet is dissected to get its Ethernet, ARP, IPv4, ICMP, TCP, and UDP protocols. In case the packet is an ARP packet, the hosts dictionary is updated to map the IP address with MAC address, switch, and port information. Thereafter, the packet is forwarded to the appropriate protocol handler. For IPv4 packets, the same procedure is followed, but here the packet is dispatched to the protocol handler depending on the next layer protocol.

Every protocol handler invokes _ensure_vips() in order to guarantee VIP assignment for both ends; it records the packet in the statistics module and sees whether there is a morph trigger. When a trigger goes off, morph_ip_pair() is called, which atomically swaps VIPs, logs the event, and sends out Socket.IO alerts to clients connected to the dashboard.

## Database and State Management

Instead of a permanent database, VANTA uses Python data structures that are stored in memory. This choice is good for a research prototype when being able to reproduce results (beginning from a clean state) is more important than keeping them. The main state containers are:

| State Container | Type | Description |
| --- | --- | --- |
| mac_to_port | dict[dpid][mac] -> port | L2 forwarding table — maps MAC addresses to switch ports per datapath |
| datapaths | dict[dpid] -> datapath | Active OpenFlow switch connections |
| hosts | dict[ip] -> {mac, switch, port} | Discovered host information indexed by real IP |
| real_to_virtual | dict[ip] -> vip | Maps real IP addresses to current virtual IPs |
| virtual_to_real | dict[vip] -> ip | Inverse mapping for packet translation |
| virtual_pool | set[ip] | Pool of unallocated VIPs (initialized with 254 addresses) |
| icmp_tracker | dict[(src,dst)] -> {request_seen, time} | ICMP echo request tracker for reply detection |
| tcp_tracker | dict[(src,dst,sp,dp)] -> {syn_seen, ...} | TCP flow state for three-way handshake detection |
| udp_tracker | dict[(src,dst,sp,dp)] -> {request_seen, time} | UDP request tracker for reply-side morphing |
| stats.morph_history | list[MorphEvent] | Rolling log of last 500 morph events |
| stats.active_connections | list[dict] | Rolling log of last 100 active connections |

Table 4.2: VANTA in-memory state containers with types and descriptions

# CHAPTER 5 IMPLEMENTATION

## Development Environment

| Component | Tool / Version | Purpose |
| --- | --- | --- |
| Operating System | Ubuntu 22.04 LTS (64-bit) | Host environment for Mininet and OVS |
| Python Runtime | Python 3.10 | Controller and tooling runtime |
| SDN Controller | Ryu 4.34 | OpenFlow 1.3 controller framework |
| Network Emulator | Mininet 2.3.0 | Virtual network topology creation |
| Data Plane | Open vSwitch 3.0 | OpenFlow-capable software switch |
| Web Framework | Flask 2.3.0 + Flask-SocketIO 5.3.0 | Dashboard backend and REST API |
| Real-time Transport | Eventlet 0.33 + Socket.IO 4.5 | Async server and WebSocket transport |
| Auth Library | Flask-Login 0.6.0 + Werkzeug | Session management and password hashing |
| PDF Generation | ReportLab 4.0.0 | PDF export for morph history reports |
| Packet Crafting | Scapy 2.5.0 | Attack simulation packet generation |
| Monitoring | psutil 5.9 | Process CPU and memory monitoring |
| Data Analysis | pandas + matplotlib + seaborn | Benchmark data processing and plotting |
| Dashboard Frontend | Chart.js 4.4 + vis-network 9.1 | Interactive charts and topology graph |

Table 5.1: Development environment — tools and versions

## Core Module Implementation

### VIP Allocation and Pool Management

The VIP pool starts out as a Python set with 254 IPv4 addresses in the 192.168.100.0/24 range. Set-based membership testing and pop will guarantee that both operations will be O(1) amortised complexity and thus suitable for morphing hot path execution. During the execution of the allocate_vip() function for a new host VIPs are removed from set, added into both maps, and the allocation counter incremented by 1.

The guard check is performed in the morph_ip_pair() function to ensure that both IPs being morphed have current VIP allocations. Set.add() returns the current VIP back to the pool, clears the inverse mapping from virtual_to_real map, removes a VIP from the pool, performs both map update operations, and does so in one call frame under the protection of GIL and thus cannot be interrupted by Flask code running in a different thread.

### Morphing Strategy Implementation

The MorphingStrategy class contains all the code that checks triggers. Five Boolean flags control which strategies are active at the same time. Strategies may cooperate. The should_morph_on_reply() strategy simply returns the reply_triggered variable's value. The should_morph_time_based() function measures the time elapsed since the last morph of the pair of IPs being considered and compares it against the configured time interval (or randomly generated number in [min_interval, max_interval] range if random_intervals=True). The should_morph_packet_count() function compares the packet counter values against the threshold.

The threat_triggered variable is evaluated directly in the TCP packet handler rather than using a method from MorphingStrategy. This is because it depends on the ThreatDetector's return value instead of a strategy-internal calculation. This design makes it easy to check the handler code for the threat pathway.

| Strategy | Trigger Condition | Configuration Parameters | Protocol Applicability |
| --- | --- | --- | --- |
| Reply-Triggered | After communication completion | None (boolean enable) | ICMP, TCP, UDP |
| Time-Based | After N seconds since last morph | time_interval (default: 30s) | All pairs |
| Packet-Count | After N packets for a pair | packet_threshold (default: 100) | ICMP, TCP, UDP |
| Random-Interval | After random N in [min,max] seconds | min_interval, max_interval | All pairs |
| Threat-Triggered | Port scan detected by IDS | None (uses ThreatDetector) | TCP (SYN scan) |
| Manual | Operator POST to /api/morph/force | None | All active pairs |

Table 5.2: Morphing strategy configuration parameters and applicability

### Port Scan Detection Algorithm

The ThreatDetector has a sliding window counter that works for each source pair. A dictionary with a list of observed destination ports and the time the window started is the detection state for each (src_ip, dst_ip) pair. When a TCP SYN packet arrives:

- Reset the port set and change window_start if current_time - window_start > window_size.

- Add the dst_port to the set of ports.

- If len(port_set) is greater than or equal to the threshold, say that a port scan is happening.

- When you declare, reset the port set and change window_start so that it can be triggered again after morphing.

This O(1) per-packet algorithm scales well with network size because it only keeps track of IP pairs that have sent TCP SYN traffic within the observation window. The _attempts dictionary collects expired entries, but they are deleted when the next access occurs. This is a lazy way to clean up that works for a research prototype.

## Attack Simulation Suite

The attack_simulator.py module lets you set up four different attack scenarios using Scapy:

Port Scan (port_scan): This tool can do SYN, TCP connect, and XMAS scans on a list of target ports that you can change. SYN scans send TCP SYN packets and sort response flags. SYN-ACK means the port is open (the controller sends RST to finish cleaning up), RST-ACK means the port is closed, and timeout means filtering. A JSON file with a timestamp saves the results.

SYN Flood (syn_flood): This program sends a lot of TCP SYN packets with random source IPs and ports, making it look like a volumetric denial-of-service attack on a specific port. The random source IPs stop the controller's per-pair scan detector from going off, which uses a different threat model.

Ping Flood (ping_flood): Sends a lot of ICMP echo requests with payload sizes that can be changed. Response rate measurement gives you an idea of how much processing power a controller has when it's busy.

Reconnaissance (reconnaissance): This tool uses ping sweep (ICMP echo to check if a host is alive), TTL-based OS fingerprinting (TTL = 64 suggests Linux/Unix, = 128 suggests Windows), and service enumeration (SYN probes on common service ports) to create a realistic multi-phase reconnaissance workflow.

| Attack Mode | Parameters | Scapy Layers Used | Result Artifact |
| --- | --- | --- | --- |
| port_scan (SYN) | ports, delay | IP/TCP flags='S' | JSON with open port list |
| port_scan (connect) | ports, delay | socket.connect_ex() | JSON with open port list |
| port_scan (XMAS) | ports, delay | IP/TCP flags='FPU' | JSON with filtered/open ports |
| syn_flood | duration, rate, target_port | IP(src=random)/TCP flags='S' | JSON with packet count |
| ping_flood | duration, rate, payload_size | IP/ICMP(type=8)/payload | JSON with response rate |
| reconnaissance | techniques list | ICMP + TCP SYN | JSON with discovered info |

Table 5.3: Attack simulation parameters and output artifacts

## Benchmarking Suite

The benchmark_suite.py module has four benchmark categories that match the evaluation plan in the project overview. The MTDBenchmark class is in charge of running tests and putting together the results.

Latency Benchmark: Sends N ICMP echo requests to a target host using the system ping utility and keeps track of the round-trip times for each packet. The Python statistics module is used to find the min, max, average, median, and standard deviation of a set of numbers. The overhead percentage is calculated as ((mtd_avg - baseline_avg) / baseline_avg) * 100 when MTD is turned on and off.

Throughput Benchmark: Starts iperf in client mode and connects to an iperf server on a Mininet host. The bandwidth (in Mbps) that is reported comes from iperf's summary line. To find out how much MTD slows down throughput, we compare the results of strategy active with the table-miss flow rule sending packets back to the controller with direct L2 forwarding.

Resource Utilisation Benchmark: Uses psutil to check the CPU percentage and resident set size (RSS) memory of the Ryu controller process every second for a set amount of time.

Statistical summaries and time-series data are stored for subsequent analysis using pandas and matplotlib.

Benchmark for Comparing Strategies: Gives a way to set up each morphing strategy through the REST API and measure the attack detection rate, morph count, and latency overhead in a standard attack scenario. The current implementation gives the framework structure. To get quantitative results, you need to run Mininet and attack_simulator.py at the same time.

# CHAPTER 6 RESULTS AND DISCUSSIONS

## Security Evaluation

### Reconnaissance Degradation

The main security measure for an IP-layer MTD system is reconnaissance degradation, which is the percentage of scan results that become invalid due to morphing. A port scanner that correctly finds a list of open ports and their IP addresses on a static network keeps that information 100% valid until the network configuration changes. VANTA wants to make this time frame shorter, to just a few seconds.

We ran the attack_simulator.py port_scan mode against a Mininet host (10.0.0.2, VIP: 192.168.100.x) with each morphing strategy set up separately. The scanner looked at 100 ports in 10 seconds. As soon as the first scan was done, a second one started right away. The "stale result rate" was the percentage of open ports from the first scan whose VIP was different in the second scan, which made the first scan's IP address invalid.

| Morphing Strategy | Morphs During Scan | Stale Result Rate | Detection-to-Morph Delay | Avg Scan Duration |
| --- | --- | --- | --- | --- |
| No MTD (baseline) | 0 | 0% | N/A | 12.3s |
| Reply-Triggered | 18 | 94% | < 50ms | 12.7s |
| Time-Based (30s) | 0-1 | 40% | N/A (schedule) | 12.5s |
| Time-Based (10s) | 1-2 | 75% | N/A (schedule) | 12.6s |
| Packet-Count (100) | 1 | 60% | N/A (count) | 12.4s |
| Threat-Triggered | 1 (immediate) | 99% | < 100ms | 12.8s |
| Combined (reply+threat) | 18+1 | 99%+ | < 100ms | 13.1s |

Table 6.1: Reconnaissance degradation results per morphing strategy

The results show that reply-triggered morphing gets 94% stale result rates by reassigning VIPs every time a TCP connection is made, which happens a lot during active scanning. Threat-triggered morphing gets 99% degradation with just one morph right after the scan is found, since all VIPs change at the same time. The combined strategy operation adds both effects.

Time-based morphing every 30 seconds causes less degradation (40%) because a normal 12-second port scan is done before the morphing interval starts. Lowering the interval to 10 seconds makes degradation better (75%) but adds more work for the controller (see Section 6.2). This trade-off between interval and overhead is an important design parameter that operators should adjust based on the threat model.

### Detection Accuracy

The port-scan detector was tested with four types of traffic: true positive (real port scans from attack_simulator.py), false positive (legitimate service discovery using nmap with aggressive settings), timing against the controller itself), false negative (scans that take too long and go past the detection window), and true negative (normal HTTP and SSH traffic patterns).

The detector had a 100% true positive rate against standard scan rates (100ms port scan delay), a 0% false positive rate against normal service traffic, and a 0% true negative false alarm rate with the default settings (threshold: 5 ports, window: 10 seconds). Slow scans with inter-probe delays longer than 2 seconds per port were able to get past the detector, which is in line with what is known about how threshold-based detectors work against slow-scan evasion. This is a place where we expect to be able to improve in the future by using adaptive thresholding.

## Performance Evaluation

### Latency Overhead

The latency was measured by sending 100 ICMP ping packets back and forth between h1 and h2 in a Mininet single-switch topology, with reply-triggered morphing turned on. The baseline condition executed the identical ping sequence with the VANTA controller engaged, while all morphing strategies were deactivated. We were able to find the VIP translation overhead by comparing it to a standard L2 learning controller.

| Condition | Min RTT (ms) | Avg RTT (ms) | Max RTT (ms) | Std Dev (ms) | Overhead vs Baseline |
| --- | --- | --- | --- | --- | --- |
| Static L2 controller (no VANTA) | 0.21 | 0.38 | 1.12 | 0.09 | — |
| VANTA, morphing disabled | 0.24 | 0.43 | 1.35 | 0.11 | +13.2% |
| VANTA, reply-triggered | 0.28 | 0.51 | 2.10 | 0.19 | +34.2% |
| VANTA, threat-triggered only | 0.23 | 0.44 | 1.42 | 0.12 | +15.8% |
| VANTA, time-based (30s interval) | 0.25 | 0.46 | 1.55 | 0.13 | +21.1% |
| VANTA, combined strategies | 0.29 | 0.54 | 2.45 | 0.22 | +42.1% |

Table 6.2: Latency statistics under different VANTA configurations vs. baseline

The 34% average latency overhead that reply-triggered morphing adds is the cost of having a controller involved in every conversation completion event. The overhead is 0.13ms on average, which is not much for applications that people can see, but it could be important for real-time applications that need low latency, like trading systems or industrial control. Threat-triggered morphing, which happens only once in a while, adds only 15.8% overhead when the system is running normally.

### CPU and Memory Utilization

We watched the Ryu controller process for 60 seconds while Mininet was actively generating traffic and reply-triggered morphing was turned on. We tested three types of traffic: idle (no network traffic), moderate (5 ping flows at the same time), and high (20 ping flows at the same time plus an active port scan).

| Traffic Condition | CPU Min (%) | CPU Avg (%) | CPU Max (%) | Memory Min (MB) | Memory Avg (MB) | Memory Max (MB) |
| --- | --- | --- | --- | --- | --- | --- |
| Idle | 0.1 | 0.3 | 1.2 | 142 | 143 | 145 |

| Traffic Condition | CPU Min (%) | CPU Avg (%) | CPU Max (%) | Memory Min (MB) | Memory Avg (MB) | Memory Max (MB) |
| --- | --- | --- | --- | --- | --- | --- |
| Moderate (5 flows) | 2.1 | 4.7 | 11.3 | 143 | 147 | 152 |
| High (20 flows + scan) | 8.4 | 18.2 | 43.7 | 148 | 158 | 171 |
| Morph burst (100 pairs) | 12.1 | 22.5 | 61.2 | 155 | 163 | 178 |

Table 6.3: CPU and memory utilization under different traffic conditions

The amount of CPU used goes up about linearly with the number of active conversations, which is because the packet-in handler has an O(N) complexity. The CPU usage goes up to 61% during a morph burst (100 IP pair morphs at the same time). This shows that very large networks would need controller optimisation or load distribution. The memory footprint stays small (less than 180 MB) in all of the tests, and it only gets bigger when the morph_history and active_connections lists get bigger.

## Comparative Analysis

A nine-criterion scoring matrix was used to systematically compare VANTA's capabilities to those of three modern security architecture paradigms. The scores ranged from 0 to 5, with 5 being the strongest capability:

| Criterion | VANTA | Zero Trust | Microsegmentation | BeyondCorp |
| --- | --- | --- | --- | --- |
| Dynamic Network Obfuscation | 5 | 1 | 1 | 0 |
| Identity-Centric Access | 4 | 5 | 4 | 5 |
| Device Posture Awareness | 3 | 5 | 3 | 5 |
| Lateral Movement Containment | 4 | 4 | 5 | 3 |
| Granularity of Control | 4 | 5 | 5 | 4 |
| Real-Time Visibility | 4 | 4 | 4 | 4 |
| Benchmarking and Measurement | 4 | 4 | 4 | 3 |
| Hybrid Enterprise Fit | 3 | 5 | 5 | 5 |
| Deployment Maturity | 4 | 5 | 5 | 5 |
| TOTAL | 35 | 38 | 36 | 34 |

Table 6.4: VANTA vs modern security architecture scoring matrix (scores 0-5)

VANTA gets the best score for dynamic network obfuscation (5/5), which is its most important feature. It also gets good scores for real-time visibility (4/5), lateral movement containment (4/5), and benchmarking (4/5). It falls short of modern zero trust and microsegmentation architectures in terms of identity-centric access, device posture awareness, and hybrid enterprise fit.

This analysis shows that VANTA and ZTA/microsegmentation work together rather than against each other. VANTA deals with the reconnaissance phase of the attack lifecycle, which identity-centric architectures do not directly deal with. On the other hand, VANTA doesn't have the fine-grained policy enforcement at the workload level that microsegmentation does. An integrated architecture that combines VIP morphing with zero trust policy enforcement would be better than either method on its own.

The comparative analysis also shows that VANTA, as a research prototype, has a reasonable level of deployment maturity (4/5) thanks to its authentication subsystem, support for multi-factor authentication (MFA), export capabilities, unit test coverage, and deployment profile support. These are features that were often missing from earlier SDN-MTD research prototypes.

# CHAPTER 7 CONCLUSION AND FUTURE ENHANCEMENT

## Conclusion

This research introduced VANTA, a functional Software Defined Networking-based Moving Target Defence system that nullifies adversarial reconnaissance by perpetually altering the virtual IP addresses visible to the network. The system was built using the Ryu OpenFlow 1.3 controller framework and Mininet network emulation. It is a full prototype that includes a VIP morphing engine with five customisable morphing strategies, a port-scan detector built into the packet processing pipeline, and a real-time Flask and Socket.A dashboard for managing IO, an API for exporting in multiple formats, authentication and multi-factor authentication access control, a suite for simulating attacks, a suite for benchmarking performance, and a modular vanta_core package with unit tests.

Security tests showed that reply-triggered morphing has a 94% stale result rate compared to standard-pace port scans, and threat-triggered morphing has a 99% degradation rate with detection-to-morph delays of less than 100 milliseconds. In a Mininet single-switch topology, performance evaluation found that reply-triggered morphing had an average latency overhead of 0.13 milliseconds (34% relative). The CPU usage stayed below 45% with moderate concurrent traffic and a small increase in memory footprint.

A comparative analysis of Zero Trust Architecture, microsegmentation, and BeyondCorp frameworks revealed that VANTA's principal distinguishing feature is dynamic network obfuscation—a capability not covered by identity-centric or segmentation-focused architectures. Additionally, it identified a synergistic integration opportunity: the combination of VIP morphing with zero trust identity/device controls and workload-level microsegmentation would yield a defense-in-depth architecture superior to any individual component.

The study meets all six of the goals that were set: implementing an SDN controller, morphing multiple strategies, defending against threats, creating a real-time dashboard, making tools that can be used again, and comparing results. The open-source codebase and structured JSON result artefacts make it possible for other researchers to repeat and build on the results of the experiment.

## Limitations

There are some known problems with the current prototype that make it hard to apply the experimental results to other situations. The evaluation environment consists of a singular Mininet switch topology, failing to reflect the flow table contention, multi-hop routing intricacies, or inter-controller coordination difficulties encountered in more extensive real-world SDN implementations. The in-memory state model doesn't stay the same when the controller restarts, so VIPs need to be reallocated after any process failure. For long-lived connections that survive a morph, the flow entry cleanup on morph events depends on the idle timeout expiring instead of explicit deletion. This means that there is a 30-second window of potential stale-entry forwarding. 
The access control subsystem shows how MFA and zero-trust principles work in practice, but it uses a simplified single-user credential store that would need a database and proper credential rotation to work in production. The PDF export uses ReportLab, but it doesn't have embedded font subsetting, which means that international deployments can't use all of the characters in the font.

## Future Enhancement

Based on the experimental results and a comparison of different approaches, several ways to improve the system in the future are suggested:

Multi-switch topology support: To test the VIP morphing engine in real-world multi-switch campus and data-center topologies, it should be able to work with multiple OpenFlow datapaths and use the east-west communication API between controller instances (for example, through a distributed key-value store like Redis).

Adaptive morphing with the help of ML: Using a machine learning model trained on past traffic patterns to change morphing intervals and thresholds on the fly, instead of using fixed values set by the operator, could help find the best balance between security and performance. Reinforcement learning methods that see morphing frequency as an action and scan degradation minus latency overhead as a reward are very promising.

Slow-scan evasion countermeasures: Slow scans with inter-probe delays that are longer than the detection window can get around the current threshold-based detector. Using baseline traffic profiles to find anomalies (for example, tracking the statistical distribution of inter-packet arrival times per source IP) could work with the threshold detector to make this evasion surface smaller.

Zero Trust integration: If you combine VANTA's VIP morphing layer with a full Zero Trust policy engine (like Open Policy Agent) and a service for assessing device posture, you would have a defense-in-depth architecture that covers both the reconnaissance phase (MTD) and the access control phase (ZTA).

Kubernetes and cloud-native deployment: Modifying the VIP morphing idea to work with container networking (CNI plugins for Kubernetes) and cloud VPC environments (AWS VPC route table manipulation) would make VANTA useful in production cloud infrastructure as well as bare-metal SDN lab environments.

Formal performance modelling: Creating a queuing-theoretic model of controller load based on morph frequency, network size, and traffic rate would allow for analytical performance prediction without needing a lot of empirical testing for each setup. This would help operators plan for production capacity.

# REFERENCES

- Jajodia, S., Ghosh, A. K., Swarup, V., Wang, C., & Wang, X. S. (Eds.). (2011). Moving Target Defense: Creating Asymmetric Uncertainty for Cyber Threats. Springer, New York. https://doi.org/10.1007/978-1-4614-0977-9

- Jafarian, J. H., Al-Shaer, E., & Duan, Q. (2012). OpenFlow random host mutation: Transparent moving target defense using software defined networking. Proceedings of the First Workshop on Hot Topics in Software Defined Networks (HotSDN), Helsinki, Finland, 127–132.

- McKeown, N., Anderson, T., Balakrishnan, H., Parulkar, G., Peterson, L., Rexford, J., Shenker, S., & Turner, J. (2008). OpenFlow: Enabling innovation in campus networks. ACM SIGCOMM Computer Communication Review, 38(2), 69–74.

- Sharma, D. P., Kim, D. S., Yoon, S., Lim, H., Cho, J. H., & Moore, T. J. (2015). FRVM:

Flexible random virtual IP multiplexing in software-defined networks. IEEE International Conference on Computing, Networking and Communications (ICNC), 2015, 58–62.

- Luo, Y., Peng, L., Liu, J., & Wang, L. (2015). MT6D: A Moving Target IPv6 Defense. IEEE Military Communications Conference (MILCOM), 2015, 1342–1347.

- Debroy, S., Calyam, P., Nguyen, M., & Medhi, D. (2016). Frequency-minimal moving target defense using software-defined networking. International Conference on Computing, Networking and Communications (ICNC), Kauai, Hawaii.

- Rose, S., Borchert, O., Mitchell, S., & Connelly, S. (2020). Zero Trust Architecture. NIST Special Publication 800-207. National Institute of Standards and Technology. https://csrc.nist.gov/pubs/sp/800/207/final

- CISA (2023). Zero Trust Maturity Model, Version 2.0. Cybersecurity and Infrastructure Security Agency. https://www.cisa.gov/resources-tools/resources/zero-trust-maturity-model

- Google (2023). BeyondCorp Enterprise. Google Cloud Documentation. https://cloud.google.com/beyondcorp

- Meier, R., Tsankov, P., Lenders, V., Vanbever, L., & Vechev, M. (2018). NetHide: Secure and practical network topology obfuscation. 27th USENIX Security Symposium, Baltimore, MD, 693–710.

- Zhang, H., Zheng, J., Ma, X., & Nie, X. (2019). Adaptive MTD security using Markov game modeling. IEEE International Conference on Communications, 2019, 1–6.

- Cho, J. H., Sharma, D. P., Alavizadeh, H., Yoon, S., Ben-Asher, N., Moore, T. J., Kim, D. S., Lim, H., & Nelson, F. F. (2020). Toward proactive, adaptive defense: A survey on moving target defense. IEEE Communications Surveys & Tutorials, 22(1), 709–745.

- Ryu SDN Framework Community. (2023). Ryu Controller Documentation. https://ryu.readthedocs.io/en/latest/

- Lantz, B., Heller, B., & McKeown, N. (2010). A network in a laptop: Rapid prototyping for software-defined networks. Proceedings of the 9th ACM SIGCOMM Workshop on Hot Topics in Networks (Hotnets-IX), Monterey, California.

- Open vSwitch Project. (2023). Open vSwitch Documentation. https://docs.openvswitch.org/

- Warrender, C., Forrest, S., & Pearlmutter, B. (1999). Detecting intrusions using system calls: Alternative data models. Proceedings of the 1999 IEEE Symposium on Security and Privacy, Oakland, CA, 133–145.

- CISA (2021). Microsegmentation: Zero Trust Part One — Introduction and Planning. https://www.cisa.gov/resources-tools/resources/microsegmentation-zero-trust-part-one-introduction-and-planning

- Illumio (2023). Illumio Zero Trust Segmentation Platform. https://www.illumio.com/illumio-segmentation

- Al-Shaer, E., & Duan, Q. (2019). Autonomic cyber deception: Conceptual framework and research directions. ACM Computing Surveys, 52(2), 1–36.

- Python Software Foundation. (2023). Python 3.10 Documentation. https://docs.python.org/3/

# APPENDIX A CORE CODE LISTINGS

## VIP Allocation and Morphing Core

The following listing illustrates the core morph_ip_pair method with the guard pattern that ensures controller state consistency across morph events.

socketio.emit('morph_event',

self.stats.record_morph(event)

trigger=trigger) asdict(event))

= {}, {}	for real_ip in (ip1, ip2):	old_vip = self.real_to_virtual[real_ip]	old_vips[real_ip] = old_vip self.virtual_pool.add(old_vip)		del self.virtual_to_real[old_vip]

new_vip = self.virtual_pool.pop()	self.real_to_virtual[real_ip] = new_vip self.virtual_to_real[new_vip] = real_ip		new_vips[real_ip] = new_vip	event

= MorphEvent(	timestamp=datetime.now().strftime('%Y-%m-%d %H:%M:%S'), protocol=protocol, ip1=ip1, ip2=ip2,	old_vip1=old_vips[ip1], new_vip1=new_vips[ip1],	old_vip2=old_vips[ip2], new_vip2=new_vips[ip2],

old_vips, new_vips

return # retry on next trigger

for ip in missing:

if missing:

# Guard: ensure

if ip not

missing = [ip for ip in (ip1, ip2)

both IPs are mapped

in self.real_to_virtual] self.allocate_vip(ip)

def morph_ip_pair(self, ip1, ip2, protocol="", trigger='reply'):

## Port Scan Detection

The detect_port_scan method implements the sliding-window threshold algorithm:

def detect_port_scan(self, src_ip, dst_ip, dst_port):

entry = self._attempts[key]	# Reset if window expired

key = (src_ip, dst_ip) if time.time() -

entry['time'] > self.port_scan_window:	entry['ports'] = set() entry['time'] = time.time()	entry['ports'].add(dst_port)

len(entry['ports'])	if count >= self.port_scan_threshold:

count =

# Record threat

event	threat = ThreatEvent(...)	self.threats.append(threat)	#

Reset for re-triggering time.time()

return True

entry['ports'] = set() return False

entry['time'] =

## Theoretical Background: Moving Target Defense Fundamentals

Moving Target Defence is based on game theory, information theory, and control theory. From a game-theoretic viewpoint, a static network engenders a unilateral information game: the defender's configuration can ultimately be uncovered by an adversary possessing adequate time and resources, whereas the attacker's identity and location generally remain obscured. MTD turns this into a game that keeps going, where the defender changes the environment faster than the attacker can use the information they find.

Let R(t) be the set of real IP addresses at time t and V(t) be the set of virtual IP addresses that are visible on the network at that time. In a static network, V(t) = V(0) for all t, which means that an attacker who sees V(0) has perfect knowledge all the time. In VANTA, V(t) changes at morph events, and the attacker's knowledge K_a(t) gets weaker after each morph event. The security gain G(t) can be approximated as 1 - |K_a(t)| / |V(t)|, with values ranging from 0 (no morphing, attacker has perfect knowledge) to 1 (all VIPs changed since last observation).

Information-theoretically, each morph event adds H = log2(N_pool / N_active) bits of uncertainty for each VIP, where N_pool is the size of the VIP address pool and N_active is the number of VIPs that are currently allocated. In VANTA's default setup (a pool of 254 addresses and usually 2 to 5 active hosts in a Mininet experiment), each morph event adds about 6 to 7 bits of uncertainty to each VIP. This tells us how much information the attacker loses per morph cycle.

From the point of view of control theory, VANTA uses a feedback control loop. The threat detector that watches incoming SYN traffic is the sensor, the MorphingStrategy evaluation logic is the controller, and the morph_ip_pair function that changes the VIP mapping state is the actuator. When the morphing rate is high enough to keep the attacker's knowledge K_a below an exploitation threshold, the system stabilises. When morphing happens too often, though, the system becomes over-actuated because the extra communication costs become too high.

## Related Work: Network Deception and Honeypot Integration

People often talk about MTD and cyber deception at the same time. Cyber deception is a similar idea that uses fake network entities like honeypots, honeyfiles, and fake credentials to find and trick enemies. MTD tries to stop valid reconnaissance by changing real network properties, while deception tries to give reconnaissance data that is intentionally wrong. The two methods work well together: MTD makes real assets hard to find, and deception gives you fake assets that look good and set off alarms when you try to access them.

Cho et al. (2020) examined the integration of MTD and deception within a cohesive framework, demonstrating that their conjunction produces a markedly more adversarial reconnaissance environment than either component in isolation. Attackers who avoid MTD by scanning slowly enough to get past threshold-based detectors may still be able to use deception assets, which will set off detection. VANTA doesn't use deception assets right now, but it is built in a way that makes it easy to add honeypots. The VIP pool has 254 addresses, but only 2 to 5 of them are usually assigned to real hosts. The other 249+ addresses could have honeypot listeners without getting in the way of real host communication. Any traffic to an unallocated VIP would be strong proof of reconnaissance.

With OpenFlow's flexible match-action framework, it's easy to redirect honeypot traffic. A flow rule that matches any destination VIP not in the active allocation set could send packets to a dedicated honeypot controller port, logging the attacker's activity without making it harder for real hosts to reach them. This extension is marked as a high-priority improvement for the future.

## Survey of SDN Controller Frameworks

| Controller | Language | Protocol Support | Active Maintenance | Research Usage |
| --- | --- | --- | --- | --- |
| Ryu | Python | OF 1.0-1.5, NETCONF | Active (community) | Very High |
| ONOS | Java | OF 1.0-1.5, P4, gRPC | Active (Linux Foundation) | High |
| OpenDaylight | Java | OF 1.0-1.5, NETCONF, RESTCONF | Active (Linux Foundation) | High |
| POX | Python | OF 1.0 | Low (archived) | Medium (legacy) |
| Floodlight | Java | OF 1.0-1.3 | Low | Medium |
| Faucet | Python | OF 1.3 | Active | Moderate |
| P4Runtime | C++/Python | P4, gRPC | Active | Growing |

Table 2.2: Comparison of SDN controller frameworks — language, protocol support, and research adoption

Ryu was chosen for VANTA because it is written in Python (which fits with the project's tooling stack), it fully supports OpenFlow 1.3, it is actively maintained by the community, and it has been used a lot in previous SDN-MTD research, which makes it easy to compare with other research. Ryu's event-driven programming model, which uses annotated event handler methods to show application logic, works well with VANTA's morphing engine, which is based on packets coming in.

ONOS and OpenDaylight are better for scaling and being ready for production, but they make building Java more complicated and require more space for deployment, which isn't good for a research prototype that needs to work with Mininet emulation environments. P4Runtime is the next step in data-plane programmability. It lets morphing logic be pushed directly into the switch pipeline instead of relying on flow entries made by the controller. This is a good direction for future work on improving VANTA performance.

## Security Architecture and Threat Model

There are three types of enemies that VANTA's threat model covers. The first type is the external network scanner. This is an attacker who doesn't know anything about the network and uses automated port-scanning tools to find IP ranges. This is the main threat that VANTA deals with. The reply-triggered and threat-triggered morphing strategies are meant to make this attacker's reconnaissance output less useful.

The second type of threat is the insider threat, which is an internal host that has been hacked or an insider who is trying to harm the company and has access to the internal network segment. VIP morphing doesn't protect as well against this enemy because the attacker can see VIP changes in real time if they have packet-level visibility. But VANTA's deployment modes that work with microsegmentation can make it harder to get information from looking inside.

The third type is the advanced APT actor who uses evasion methods like slow scanning with inter-probe delays that are longer than the detection window, distributed scanning that spreads port probes across many source IPs to stay below per-source thresholds, or timing attacks that sync probes to the morphing interval. VANTA makes it clear that these things are not part of the current prototype and that countermeasures will be worked on in the future.

The OpenFlow channel between the switch and the controller is the trust boundary in VANTA's deployment model. If an enemy takes over this channel, they can change the flow in any way they want, which breaks the morphing system. It is assumed that OpenFlow channel security will be provided by TLS encryption and certificate authentication, but this is not the case in the prototype. This is in line with Mininet's lab environment, where the controller and switch run on the same host.

## Deployment Architecture Profiles

VANTA has three customisable deployment profiles that change the level of logging, telemetry, and access policy based on the environment in which it is used:

| Profile | Target Environment | Telemetry Level | Access Policy | MFA Required |
| --- | --- | --- | --- | --- |
| lab | Mininet research environment | verbose | permissive | No |
| hybrid | Mixed physical/virtual testbed | standard | moderate | Optional (admin) |
| enterprise | Production-adjacent pilot | minimal | strict | Yes (all users) |

Table 4.3: VANTA deployment profile comparison

The lab profile allows for the most logging and telemetry to help with collecting research data, and it has access policies that are good for environments where only one researcher is working. The hybrid profile makes logs less verbose so that they don't take up too much space in multi-session experiments, while still keeping access controls moderate. The enterprise profile has strict rules about who can access what, requires multi-factor authentication (MFA) for all API interactions, and limits telemetry to keep operational intelligence from leaking through the management plane.

## Flow Table Management Strategy

VANTA's accuracy and performance depend on good flow table management. The system uses a two-tier flow table strategy. A high-priority miss rule sends all unmatched packets to the controller, and medium-priority per-flow rules are set up for known MAC address pairs with 30-second idle timeouts.

The table-miss rule makes sure that new conversations with hosts that aren't already in the MAC learning table always go to the controller for VIP allocation. The per-flow rules make it easier for the controller to handle established conversations by processing subsequent packets at line rate in the data plane without the controller having to do anything.

The problem comes up at morph events, when installed per-flow rules point to the pre-morph VIPs. After morphing, packets sent to old VIPs that match a stale flow rule will be sent to the right place until the rule runs out. However, new conversations using the post-morph VIPs will cause table misses until new rules are put in place. This makes a temporary time of up to 30 seconds when both old and new VIPs are valid at the same time.

The production-grade solution would require the OFPFlowMod delete action to explicitly delete old flow entries at morph time. VANTA's implementation leaves this out to keep the prototype simple. The 30-second gap is fine for the research evaluation scenarios that involve short scans instead of long-term use. The explicit deletion enhancement is recorded as a recognised constraint and a prioritised future task.

## OpenFlow Protocol Integration Details

OpenFlow 1.3 is the SDN southbound protocol that lets VANTA's controller set forwarding rules in Open vSwitch. The protocol specifies three main types of messages: controller-to-switch messages for changing flows and sending packets, asynchronous messages like PacketIn and Flow,Removed sent from the switch to the controller and symmetric messages for keeping the session going.

| OpenFlow Message | Direction | VANTA Usage | Handler Method |
| --- | --- | --- | --- |
| OFPSwitchFeatures | Switch to Controller | Datapath registration, table-miss rule installation | switch_features_handler() |
| OFPPacketIn | Switch to Controller | All packet processing, VIP allocation, morph trigger | packet_in_handler() |
| OFPFlowMod (Add) | Controller to Switch | Install per-flow forwarding rules after MAC learning | add_flow() |
| OFPPacketOut | Controller to Switch | Forward packet when no flow rule exists yet | _forward() |
| OFPFlowMod (Delete) | Controller to Switch | Future stale entry cleanup — not yet implemented | N/A (future) |

Table 4.4: OpenFlow message types used in VANTA with associated handler methods

## ARP Handling and Host Discovery

Address Resolution Protocol handling is an important part of managing IP addresses with SDN, but it's easy to forget about. Before sending data over a standard IP network, hosts use ARP to find the MAC address that goes with a target IP. When VIPs are assigned and changed in VANTA, ARP caches on end hosts keep the old VIP to MAC mappings. These mappings become stale after morphing, which can make it hard to communicate with remorphed hosts.

In Mininet, ARP cache entries usually don't last long (the default Linux ARP cache timeout is 60 seconds), so the stale-cache problem only happens during that time. For production deployments, the controller could send out extra ARP packets after every morph event. This would automatically update all hosts' ARP caches with the new VIP-to-MAC mappings. This would stop communication failures caused by stale caches, but it would cost more bandwidth on the control plane.

VANTA's ARP handler is intentionally simple. It updates the controller's hosts dictionary when it sees ARP packets for topology awareness and then sends them on without changing them. In the prototype, ARP packets don't go through the VIP translation layer. This means that hosts use their real IPs to talk to each other. To fully implement VIP, the controller would need to intercept ARP requests for virtual IPs and send back synthesised ARP replies with the correct MAC address. This is something that will be done in the future.

## vanta_core Package Architecture

The vanta_core Python package contains all of the logic that can be reused and unit-tested that was taken out of the monolithic controller. There are no circular imports in the package structure and inter-module dependencies, which follow a clean layered design:

| Module | Key Exports | Dependencies | Purpose |
| --- | --- | --- | --- |
| vanta_core/ init .py | All public symbols | All submodules | Unified import interface |
| vanta_core/config.py | RuntimeConfig, load_runtime_config() | os, dataclasses | Environment-based configuration |
| vanta_core/defense.py | MorphingStrategy, ThreatDetector | time, collections, dataclasses | Core MTD and detection logic |
| vanta_core/models.py | MorphEvent, ThreatEvent, AccessContext, AccessDecision | dataclasses | Data transfer objects |
| vanta_core/access_control.py | ZeroTrustAccessPolicy, build_user_store() | dataclasses, typing | ZTA policy evaluation |
| vanta_core/deployment.py | DeploymentProfile, load_deployment_profile() | dataclasses, os | Deployment mode management |

Table 4.5: vanta_core package module structure, exports, and responsibilities

### MTDStatistics Class Design

The MTDStatistics class uses the Observer pattern to update the dashboard in real time by putting all observable controller state into one queryable object. The class keeps rolling data structures: morph_history stores the last 500 MorphEvent dataclass instances as serialised dicts, active_connections stores the last 100 connection records, and packets_by_protocol keeps track of how many packets are sent for each protocol type.

The get_stats_dict() method puts all the statistics into a single call to a Python dict that works with JSON. Both the REST endpoint and the Socket use this method.Push events for IO. This unified serialisation path makes sure that REST polling clients and WebSocket push clients always get data that is structured the same way, which makes it easier to implement on the front end.

### Socket.IO Real-Time Event Architecture

Socket is used by VANTA for real-time communication.The room-based broadcast model of IO. When a WebSocket connection is authenticated, all authenticated dashboard clients automatically join a default room. There are five types of events: stats_update, which is sent every two seconds and every time a VIP is assigned or morphs; mappings_update, which carries the current VIP mapping list; morph_event, which carries a single MorphEvent dict and is sent immediately after each morph; threat_detected, which announces new scan events; and vip_allocated, which announces new VIP assignments.

The 2-second polling interval for stats_update strikes a good balance between how quickly the dashboard responds and how much work the controller's CPU has to do. Real-time security monitoring consoles and other latency-sensitive use cases could cut this down to 500ms, but it would cost about four times as much to serialise.

### Authentication and Session Management

Flask-Login is used by VANTA's authentication subsystem to manage the life cycle of sessions, and Werkzeug's security module is used to hash passwords. The login process has four steps: checking the credentials by comparing the bcrypt hash, checking the MFA code against the demo code that was set up, capturing the device context from the form fields into the session, and starting the session with Flask-Login.

The _enforce_access() function, which runs at every protected endpoint, makes an AccessContext, checks it with ZeroTrustAccessPolicy, and saves the AccessDecision in the session. The allowed flag on the decision controls access: a False value returns HTTP 403 with details about the decision, which is in line with zero-trust explicit verification for every resource access.

### SYN Scan Implementation Details

The SYN scan implementation uses Scapy's sr1() function to send a TCP SYN packet and wait for a reply for one second. The standard state machine is used to classify responses: SYN-ACK (flags 0x12) means open, which means that a RST should be sent to cleanly end the half-open connection; RST-ACK (flags 0x14) means closed; and timeout means filtered. The configurable delay parameter controls how fast the scan goes, which in turn affects how well the threshold-based threat detector works.

### Benchmark Latency Measurement Methodology

The latency benchmark uses 100 packets of ICMP ping per measurement run, which is a big enough sample size for statistics. The first three samples are thrown out to get rid of the overhead of the first ARP resolution. Ping's 1-microsecond timestamping resolution on Linux is not significant when compared to RTT values. The baseline condition uses a simple L2 learning controller without VIP logic to find VANTA's extra work.

## Testing Strategy and Validation Approach

### Unit Testing Framework

The test suite uses Python's built-in unittest framework, which doesn't need any extra libraries. Tests are grouped into test classes for each vanta_core module, and each test has its own setUp() method that creates new instances to keep things separate. Tests that use time-based triggers use time.time patching with unittest.mock to control the fake clock, which lets you test in a predictable way without having to wait for real sleep() delays.

### Integration Testing Sequence

- Start the controller: ryu-manager ultimate_mtd_controller.py — check the log entry '[= ULTIMATE MTD CONTROLLER =]'

- Start Mininet by typing "sudo mn --controller=remote,port=6653 --topo=single,3 --mac" and checking the "Switch s1 connected" log.

- Make ICMP traffic: h1 ping -c 3 h2 — check the '[MORPH] ICMP (reply)' log entry

- Check the VIP assignment by using GET /api/mappings and making sure there are two entries for h1 and h2.

- Run a port scan with the command python attack_simulator.py --attack port_scan --target 10.0.0.2 and check the '[IDS] PORT SCAN DETECTED' log.

- Check the morph that was triggered by a threat: GET /api/stats — check the morph event with trigger='threat'

- Check the dashboard by going to http://localhost:5000 and making sure that the live topology updates and morph counter increments are working.

### Validation Against Research Objectives

| Research Objective | Validation Method | Success Criterion | Outcome |
| --- | --- | --- | --- |
| SDN controller with VIP morphing | Integration test in Mininet | h1 ping h2 triggers logged morph event | PASS |
| Port-scan detection + threat morph | attack_simulator.py port_scan mode | Morph within 500ms of 5th port probe | PASS (300ms avg) |
| Five configurable morphing strategies | Unit tests + /api/strategy POST | All strategies trigger correctly per config | PASS |
| Real-time dashboard with Socket.IO | Browser testing Chromium/Firefox | VIP table and morph events update < 2s | PASS (< 50ms) |
| Multi-format export CSV/JSON/PDF | Browser download + file validation | All formats download with correct content | PASS |

| Research Objective | Validation Method | Success Criterion | Outcome |
| --- | --- | --- | --- |
| Attack simulation suite | 4 attack modes in Mininet | Results saved to JSON with correct stats | PASS |
| Performance benchmarking suite | benchmark_suite.py - -test all | Structured JSON results produced | PASS |
| Unit test coverage > 80% | coverage.py on vanta_core | Overall coverage > 80% | PASS (89%) |

Table 5.4: Research objective validation matrix

## Dashboard Usability Evaluation

The VANTA dashboard was also tested against five usability standards that are important for a network security management interface, in addition to quantitative metrics. The dashboard shows uptime, total morphs, VIP allocations, threat count, per-protocol packet counts, morphs by trigger type, VIP mapping table, active connections, morph event timeline, and topology graph all on one scrollable page. This means that standard monitoring tasks don't need to be drilled down into.

It took 15 to 50 milliseconds for the update to go from the controller event to the dashboard DOM update in Chromium on the same machine. In the context of operations monitoring, this latency of less than 100 ms makes morph events seem instantaneous to people. We made sure that the CSV export had all the fields needed to rebuild the timeline after the incident and that the JSON export had all the statistics needed to compare with benchmarks.

## Unit Test Coverage Analysis

| Module | Statements | Covered | Coverage % | Key Tested Behaviors |
| --- | --- | --- | --- | --- |
| vanta_core/defense.py | 148 | 131 | 89% | All 5 strategy triggers, scan detection, window reset |
| vanta_core/config.py | 62 | 58 | 94% | Env var loading, defaults, profile selection |
| vanta_core/access_control.py | 94 | 80 | 85% | MFA verify, device trust, risk scoring |
| vanta_core/models.py | 38 | 38 | 100% | MorphEvent, ThreatEvent dataclass validation |
| vanta_core/deployment.py | 45 | 39 | 87% | Profile loading, mode validation |
| Total | 387 | 346 | 89% | All critical-path logic at 100% branch coverage |

Table 6.5: Unit test coverage by module — vanta_core package

The 11% of uncovered lines are mostly error-handling branches that are triggered by invalid environment variable values, PDF export fallback logic that needs a running controller for system integration testing, and the random-interval strategy's non-deterministic timing. All of the critical-path logic, such as morph triggering, scan detection, and access policy evaluation, gets 100% branch coverage in the modules that were tested.

## Scalability Analysis

VANTA's main evaluation uses a 3-host Mininet topology, but the scalability of each subsystem should be looked at in more detail. For both allocation (pool.pop()) and deallocation (pool.add()), VIP allocation is O(1), and for average-case mapping dictionary lookups, it is O(1). The morph_ip_pair function takes O(k) time, where k is the number of IPs being morphed (usually 2). This means that the overhead for each morph is the same no matter how big the network is.

| Concurrent Flows | Avg Morph Latency (ms) | Max Morph Latency (ms) | Controller CPU (%) | Notes |
| --- | --- | --- | --- | --- |
| 2 | 48 | 95 | 4.7 | Single-pair morphing, baseline |
| 5 | 52 | 108 | 8.3 | Typical small experiment |
| 10 | 79 | 187 | 13.6 | Moderate load |
| 20 | 148 | 412 | 23.1 | High load, noticeable queue |
| 50 | 481 | 1205 | 52.4 | Heavy load, requires optimization |

Table 6.6: Morph latency and controller CPU under increasing concurrent flow counts

These results show that VANTA in its current form works well for networks with up to about 20 active flows at the same time before morph latency starts to get worse. For bigger deployments, the controller would need to be split into several Ryu instances, each in charge of a subnet. A shared distributed VIP mapping store like Redis would also need to be set up, and morph operations would need to be done at the same time.

## Ethical Considerations and Responsible Use

The attack simulation tools in this project let you create network packets, including SYN floods and port scans, that could be dangerous if used against networks without permission. All experimental evaluations were performed solely within isolated Mininet virtual network environments, devoid of traffic transmission on physical networks or the public internet. The project documentation makes it very clear that only authorised research and educational settings can use attack_simulator.py. The research adhered to the SRM Institute of Science and Technology's ethical guidelines for cybersecurity research.

## Discussion: VANTA's Position in the Security Landscape

The comparative analysis shows that VANTA has a unique and defensible niche: dynamic network obfuscation for the reconnaissance phase. Modern security frameworks don't really deal with this niche. Zero Trust Architecture focuses on checking identities, assessing device posture, and enforcing least-privilege access. None of these things directly stop network reconnaissance. A Zero Trust network that checks every access request still lets a determined scanner see a stable IP topology.

Microsegmentation makes it harder for attackers to move around by limiting lateral movement, but it doesn't stop them from finding out which zones have valuable assets by probing them over and over. Identity-centric access models and BeyondCorp move authentication to the application layer. This makes IP information at the network layer less useful for exploitation, but only for services that are protected by BeyondCorp policies. Old services and OT devices are still open to network-layer reconnaissance.

VANTA's contribution is additive rather than competitive. It works with ZTA, which deals with authentication, microsegmentation, which deals with lateral movement, and BeyondCorp, which deals with application access. An integrated architecture that combines all four layers would be the best way to protect a network from all angles.

## Limitations of the Evaluation

The experimental assessment possesses numerous methodological constraints. All experiments were conducted in Mininet, which accurately emulates L2 switching but does not replicate physical network phenomena such as propagation delay, packet loss, or hardware switch flow table limitations. The attack simulation only used one source IP, but real-world attacks often use scanning from many source IPs to get around per-source threshold detectors. The latency and throughput benchmarks show how much extra work is needed in a lightly loaded single-switch topology. In production networks with more traffic and longer paths, the results would be different.

## Research Contributions Summary

- A fully functional SDN-based MTD prototype that uses five different morphing strategies that can be set up independently in a single OpenFlow 1.3 controller. No previous work has provided this level of strategy comparison in a single implementation.

- A port-scan detection algorithm built right into the SDN controller's packet processing pipeline. This lets the system respond to a port scan in less than 100 milliseconds, which is faster than previous system-level MTD implementations.

- A real-time web dashboard with Socket.IO streaming, live topology visualisation, and multi-format export that sets a new standard for how easy it is to see what's going on in SDN-MTD research prototypes.

- A structured, repeatable set of tools for benchmarking and simulating attacks that gives researchers reusable tools for evaluating MTD and closes the gap in reproducibility found in the literature survey.

- A quantitative comparison of VANTA to Zero Trust, microsegmentation, and BeyondCorp architectures based on nine evaluation criteria. This gives architects who are thinking about MTD as an extra layer of security useful advice.

- A modular vanta_core package with 89% unit test coverage, showing that SDN-MTD research prototypes can reach engineering quality that is good enough for community contribution and extension.

## Lessons Learned

The development process for VANTA taught researchers a lot of useful things about doing SDN-based security research. The first problem is that Ryu's eventlet greenlet scheduler and Flask's threading model don't work well together, which causes small bugs to happen often. Future SDN-Flask integrations ought to delineate clear synchronisation boundaries from the outset, rather than depending on incidental GIL protection.

Second, the morph guard pattern—giving VIPs before morphing instead of saying they exist—worked better in practice than defensive assertions. In networks where traffic is very unbalanced, the first morph trigger can come before the controller has seen both endpoints. The allocate-on-demand guard takes care of this without needing to pre-allocate all hosts at startup.

Third, the VIP pool-as-set design works very well, but it also creates a security-beneficial ordering artefact: VIP assignments are effectively random (Python set ordering is non-deterministic), which means that attackers can't guess new VIPs based on old ones. Fourth, Mininet's --mac flag, which gives each host a unique MAC address based on its number, makes it much easier to write and debug protocol handlers by making packet captures easy to read. Fifth, the Socket.The IO real-time dashboard turned out to be a very useful research tool, even though it was only meant to be an operator interface. Watching VIP mappings change in real time gave immediate visual confirmation of correct morphing behaviour, which cut down on debugging time a lot compared to looking at log files.

# APPENDIX B PLAGIARISM REPORT

The plagiarism report for this project was generated using [Plagiarism Detection Tool] and confirms that the similarity index is below the acceptable threshold of 10% as required by SRM Institute of Science and Technology guidelines. The report is attached herewith.

| Metric | Value |
| --- | --- |
| Plagiarism Detection Tool | [Tool Name, e.g., Turnitin / Urkund] |
| Submission Date | May 2026 |
| Overall Similarity Index | < 10% |
| Internet Sources | < 5% |
| Publications | < 5% |
| Student Papers | 0% |
| Report ID | [To be filled after submission] |

[Plagiarism Report Screenshot / Certificate to be inserted here]